# PCB DESIGN & MANUFACTURING DOCUMENT

# RC CAR

# 1. Scope & Objectives:

# Scope

## The scope of this project is to design and implement a simple Arduino-based PCB that integrates a battery supply, LED indicator, and push button switch. The PCB is designed using KiCad and includes schematic capture, PCB layout, and 3D visualization.

# Objectives

## To design a schematic using Arduino UNO R2 module

## To create a single-layer PCB

## To place and route components correctly

## To verify the design using Design Rule Check (DRC)

## To generate manufacturing files such as Gerber and BOM

## To visualize the PCB in 2D and 3D views

# 2. Schematic:

## UNO R2 (A1) – Acts as the main controller

## Battery (BT1) – Provides power supply

## LED (D1) – Used as a visual indicator

## Push Button Switch (SW1) – Used as user input

# 3.PCB Layout-Placement & Routing:

## Component Placement

## The Arduino UNO is placed centrally for easy access to all pins.

## The battery holder is placed at the bottom for mechanical stability.

## The push button is placed at the edge of the PCB for easy user access.

## The LED is placed near the switch for visual indication.

## Routing

## Tracks are routed carefully to avoid overlaps.

## Power and ground connections are kept short.

## Proper clearance is maintained between tracks.

## A single-layer routing approach is followed.

# 4. Footprint Assignment & PCB Design Rules (DRC):

| S.No | Component Name | Reference Designator | Schematic Symbol | Footprint Assigned | Mounting Type |
| --- | --- | --- | --- | --- | --- |
| 1 | Arduino UNO R2 | A1 | Arduino_UNO_R2 | Arduino_UNO_R2_THT | Through Hole |
| 2 | Battery Holder (2 Cell) | BT1 | Battery | Battery_Holder_2x18650 | Through Hole |
| 3 | LED | D1 | LED | LED_THT_5mm | Through Hole |
| 4 | Push Button Switch | SW1 | SW_Push | SW_Tactile_THT | Through Hole |

## The minimum track width is maintained at 0.25 mm to ensure safe current flow.

## The minimum track clearance of 0.25 mm is provided to prevent short circuits.

## The via diameter is set to 0.8 mm, which is suitable for through-hole PCB designs.

## The via drill size is maintained at 0.4 mm as a standard drilling value.

## A pad-to-pad clearance of 0.25 mm is ensured to avoid solder bridging.

## Copper is kept at least 0.5 mm away from the board edge to prevent edge damage.

## A silkscreen-to-pad clearance of 0.2 mm is maintained for clear component markings.

## The PCB is designed as a single-layer board using one copper layer.

## A solder mask clearance of 0.05 mm is provided to ensure proper soldering.

## The Design Rule Check (DRC) is successfully passed with no errors or warnings.

# Design Rule Check (DRC):

## Minimum track width maintained

## Proper clearance between pads and tracks

## No unconnected nets

## No short circuits or overlap errors

## The DRC was successfully completed without errors.

## 5. 2D/3D PCB View & Final checks:

# 2D View

## Shows accurate component placement

## Confirms routing correctness

## Verifies board outline

# 3D View

## Confirms real-world appearance

## Verifies mechanical fitting of battery holder and switch

## Helps detect height or placement issues

# 6. Production files(Fabrication & Assembly):

## The following files are generated:

## Gerber files (Top copper, silkscreen, solder mask)

## Drill files

## Pick and place files (if required)

## BOM file

## These files are sufficient for PCB manufacturing and assembly.

# 6.1 Bill Of Materials (BOM):

| ## S.No | ## Component Name | ## Reference | ## Quantity |
| --- | --- | --- | --- |
| ## 1 | ## Arduino UNO R2 | ## A1 | ## 1 |
| ## 2 | ## Battery Holder (2 Cell) | ## BT1 | ## 1 |
| ## 3 | ## LED | ## D1 | ## 1 |
| ## 4 | ## Push Button Switch | ## SW1 | ## 1 |
| ## 5 | ## PCB Board | ## — | ## 1 |

Bottom of Form